SMART IRRIGATION AND WATER MANAGEMENT SYSTEM WITH GSM ALERT

/* main.c*/

#include <LPC21xx.h>

#include "lcddisp.h"

#include "uart01.h"

#include "delay.h"

#define MOTOR (1<<4)      // ACTIVE LOW

#define SOIL  (1<<5)

unsigned char data;

int dry_flag=0;

int wet_flag=0;

int UART0_RX_CHECK(void);

int main()

{

UART0_CONFIG();

LCD_INIT();

/* MOTOR OUTPUT */

IODIR0 |= MOTOR;

/* SOIL SENSOR INPUT */

IODIR0 &= ~SOIL;

/* MOTOR OFF INITIALLY */

IOSET0 = MOTOR;

/* TITLE */

LCD_COMMAND(0x80);

LCD_STR("SMART");

LCD_COMMAND(0xC0);

LCD_STR("IRRIGATION");

delay_second(2);

LCD_COMMAND(0x01);

/* GSM INIT */

UART0_STR("AT\r\n");

delay_second(1);

UART0_STR("AT+CMGF=1\r\n");

delay_second(1);

UART0_STR("AT+CNMI=2,2,0,0,0\r\n");

delay_second(1);

while(1)

{

/* SOIL DRY */

if(((IOPIN0>>5)&1)==1)

{

if(!dry_flag)

{

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

LCD_STR("SOIL DRY");

UART0_STR("AT+CMGS=\"+918778934793\"\r\n");

delay_second(1);

UART0_STR("SOIL DRY SEND a");

UART0_TX(0x1A);

delay_second(3);

dry_flag=1;

wet_flag=0;

}

}

/* SOIL WET */

else if(((IOPIN0>>5)&1)==0)

{

if(!wet_flag)

{

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

LCD_STR("SOIL WET");

UART0_STR("AT+CMGS=\"+918778934793\"\r\n");

delay_second(1);

UART0_STR("SOIL WET SEND b");

UART0_TX(0x1A);

delay_second(3);

wet_flag=1;

dry_flag=0;

}

}

/* NON BLOCKING UART RECEIVE */

data = UART0_RX_CHECK();

/* MOTOR ON */

if(data=='a')

{

IOCLR0 = MOTOR;

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

LCD_STR("MOTOR ON");

UART0_STR("AT+CMGS=\"+918778934793\"\r\n");

delay_second(1);

UART0_STR("MOTOR ON");

UART0_TX(0x1A);

delay_second(2);

}

/* MOTOR OFF */

if(data=='b')

{

IOSET0 = MOTOR;

LCD_COMMAND(0x01);

LCD_COMMAND(0x80);

LCD_STR("MOTOR OFF");

UART0_STR("AT+CMGS=\"+918778934793\"\r\n");

delay_second(1);

UART0_STR("MOTOR OFF");

UART0_TX(0x1A);

delay_second(2);

}

}

}

/* NON BLOCKING UART RECEIVE */

int UART0_RX_CHECK(void)

{

if(U0LSR & 1)

{

return U0RBR;

}

return -1;

}

/*uart.h*/

#include <LPC21xx.h>

void UART0_CONFIG(void);

void UART0_TX(unsigned char d);

void UART0_STR(char *s);

unsigned char UART0_RX(void);

void UART0_CONFIG(void)

{

PINSEL0 |= 0x00000005;

U0LCR = 0x83;

U0DLL = 97;

U0DLM = 0;

U0LCR = 0x03;

}

void UART0_TX(unsigned char d)

{

while((U0LSR & (1<<5)) == 0);

U0THR = d;

}

void UART0_STR(char *s)

{

while(*s)

{

UART0_TX(*s++);

}

}

unsigned char UART0_RX(void)

{

while((U0LSR & 1) == 0);

return U0RBR;

}

/*lcd.h*/

#include <LPC21xx.h>

#include "delay_millisecond.h"

#define LCD_D (0xF<<14)

#define RS (1<<8)

#define E  (1<<9)

void LCD_INIT(void);

void LCD_COMMAND(unsigned char cmd);

void LCD_DATA(unsigned char d);

void LCD_STR(unsigned char *s);

void LCD_INIT(void)

{

delay_second(1);

IODIR0 |= LCD_D | RS | E;

LCD_COMMAND(0x01);

LCD_COMMAND(0x02);

LCD_COMMAND(0x0C);

LCD_COMMAND(0x28);

}

void LCD_COMMAND(unsigned char cmd)

{

IOCLR0 = LCD_D;

IOSET0 = (cmd & 0xF0)<<10;

IOCLR0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

IOCLR0 = LCD_D;

IOSET0 = (cmd & 0x0F)<<14;

IOCLR0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

}

void LCD_DATA(unsigned char d)

{

IOCLR0 = LCD_D;

IOSET0 = (d & 0xF0)<<10;

IOSET0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

IOCLR0 = LCD_D;

IOSET0 = (d & 0x0F)<<14;

IOSET0 = RS;

IOSET0 = E;

delay_millisecond(2);

IOCLR0 = E;

}

void LCD_STR(unsigned char *s)

{

int i;

for(i=0; s[i]; i++)

{

LCD_DATA(s[i]);

}

}

/*delay.h*/

#include <LPC21xx.H>

void delay_millisecond(unsigned int millisecond)

{

T0PR=15000-1;

T0TCR=0x01;

while(T0TC<millisecond);

T0TCR=0x03;

T0TCR=0x00;

}

void delay_second(unsigned int second)

{

T0PR=15000000-1;

T0TCR=0x01;

while(T0TC<second);

T0TCR=0x03;

T0TCR=0x00;

}

void delay_microsecond(unsigned int microsecond)

{

T0PR=15-1;

T0TCR=0x01;

while(T0TC<microsecond);

T0TCR=0x03;

T0TCR=0x00;

}